| Machine Type & Model |  | Machine No. | SCA |  | System | Gen1 | Gen2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mine Name |  | Mine Section & No. |  | Timesheet Ref. |  |  |  |

| INSTRUCTIONS |  | ACCEPTABLE LIMITS | COMMENTS |  |  |  |  | STATUS |  | INITIALS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | GENERAL INSPECTION OF PDS BYPASS SWITCH |  |  |  |  | Circle () if completed, or () if outstanding. |  |  |  |  |
|  | Inspect certification label. | - Note Serial # & Date - Note Certificate No. | Serial: Date: Certificate: |  |  |  |  |  |  |  |
|  | Check bolts on enclosure cover to ensure tight. | - Check bolts are firm |  |  |  |  |  |  |  |  |
|  | Check mounting bolts to ensure tight. | - Check bolts are firm |  |  |  |  |  |  |  |  |
|  | Check Cable Entries and Glands | - Correct Certification - Hose Clamps Fitted and Secure |  |  |  |  |  |  |  |  |
|  | Inspect PDS Bypass Push Button | - Operates Freely - Locked in ‘ON’ Position |  |  |  |  |  |  |  |  |
|  | GENERAL INSPECTION OF MAIN ENCLOSURE |  |  |  |  | Circle () if completed, or () if outstanding. |  |  |  |  |
|  | Inspect certification label. | - Note Serial # & Date - Note Certificate No. | Serial: Date: Certificate: |  |  |  |  |  |  |  |
|  | Check all Cable markers, Traffolyte labels fitted | - Refer Drawing |  |  |  |  |  |  |  |  |
|  | Check Relay Retaining Clips are secure | - Clips fitted |  |  |  |  |  |  |  |  |
|  | Check bolts on enclosure cover to ensure tight. | 1. Check bolts are firm |  |  |  |  |  |  |  |  |
|  | Check mounting bolts to ensure tight. | 1. Check bolts are firm |  |  |  |  |  |  |  |  |
|  | Check Cable Entries and Glands | - Correct Certification - Hose Clamps Fitted and Secure |  |  |  |  |  |  |  |  |
|  | GENERAL INSPECTION OF DISPLAY POD |  |  |  |  | Circle () if completed, or () if outstanding. |  |  |  |  |
|  | Inspect certification label. | - Note Serial # & Date - Note Certificate No. | Serial: Date: Certificate: |  |  |  |  |  |  |  |
|  | Inspect Display POD location | - LED’s Visible by Operator |  |  |  |  |  |  |  |  |
|  | Check mounting bolts to ensure tight. | - Check bolts are firm |  |  |  |  |  |  |  |  |
|  | Check Cable Entry and Gland | - Correct Certification - Hose Clamps Fitted and Secure |  |  |  |  |  |  |  |  |
|  | GENERAL INSPECTION OF FLASHER/SIREN COMBO |  |  |  |  | Circle () if completed, or () if outstanding. |  |  |  |  |
|  | Inspect certification label. | - Note Serial # & Date - Note Certificate No. | Serial: Date: Certificate: |  |  |  |  |  |  |  |
|  | Inspect Flasher/Siren Location | - LED’s not obstructed from view |  |  |  |  |  |  |  |  |
|  | Check mounting bolts to ensure tight. | - Check bolts are firm |  |  |  |  |  |  |  |  |
|  | Check Cable Entry and Gland | - Correct Certification - Hose Clamps Fitted and Secure |  |  |  |  |  |  |  |  |
|  | GENERAL INSPECTION OF PROXIMITY MODULE A1 |  |  |  |  | Circle () if completed, or () if outstanding. |  |  |  |  |
|  | Inspect certification label. | - Note Serial # & Date - Note Certificate No. | Serial: Date: Certificate: |  |  |  |  |  |  |  |
|  | Inspect Proximity Module Location | - Check Clearances around Generator >90mm - Check LED Window Visible |  |  |  |  |  |  |  |  |
|  | Check mounting bolts to ensure tight. | - Check bolts are firm |  |  |  |  |  |  |  |  |
|  | Check Cable Entry and Gland | - Correct Certification - Hose Clamps Fitted and Secure |  |  |  |  |  |  |  |  |
|  | GENERAL INSPECTION OF PROXIMITY MODULE A2 |  |  |  |  | Circle () if completed, or () if outstanding. |  |  |  |  |
|  | Inspect certification label. | - Note Serial # & Date - Note Certificate No. | Serial: Date: Certificate: |  |  |  |  |  |  |  |
|  | Inspect Proximity Module Location | - Check Clearances around Generator >90mm - Check LED Window Visible |  |  |  |  |  |  |  |  |
|  | Check mounting bolts to ensure tight. | - Check bolts are firm |  |  |  |  |  |  |  |  |
|  | Check Cable Entry and Gland | - Correct Certification - Hose Clamps Fitted and Secure |  |  |  |  |  |  |  |  |
|  | GENERAL INSPECTION OF PROXIMITY MODULE B1 |  |  |  |  | Circle () if completed, or () if outstanding. |  |  |  |  |
|  | Inspect certification label. | - Note Serial # & Date - Note Certificate No. | Serial: Date: Certificate: |  |  |  |  |  |  |  |
|  | Inspect Proximity Module Location | - Check Clearances around Generator >90mm - Check LED Window Visible |  |  |  |  |  |  |  |  |
|  | Check mounting bolts to ensure tight. | - Check bolts are firm |  |  |  |  |  |  |  |  |
|  | Check Cable Entry and Gland | - Correct Certification - Hose Clamps Fitted and Secure |  |  |  |  |  |  |  |  |
|  | GENERAL INSPECTION OF PROXIMITY MODULE B2 |  |  |  |  | Circle () if completed, or () if outstanding. |  |  |  |  |
|  | Inspect certification label. | - Note Serial # & Date - Note Certificate No. | Serial: Date: Certificate: |  |  |  |  |  |  |  |
|  | Inspect Proximity Module Location | - Check Clearances around Generator >90mm - Check LED Window Visible |  |  |  |  |  |  |  |  |
|  | Check mounting bolts to ensure tight. | - Check bolts are firm |  |  |  |  |  |  |  |  |
|  | GENERAL INSPECTION OF JUNCTIONBOX A (Front) |  |  |  |  | Circle () if completed, or () if outstanding. |  |  |  |  |
|  | Inspect certification label. | - Note Serial # & Date - Note Certificate No. | Serial: Date: Certificate: |  |  |  |  |  |  |  |
|  | Check mounting bolts to ensure tight. | - Check bolts are firm |  |  |  |  |  |  |  |  |
|  | Check Cable Entry and Gland | - Correct Certification - Hose Clamps Fitted and Secure |  |  |  |  |  |  |  |  |
|  | GENERAL INSPECTION OF JUNCTION BOX B (Rear) |  |  |  |  | Circle () if completed, or () if outstanding. |  |  |  |  |
|  | Inspect certification label. | - Note Serial # & Date - Note Certificate No. | Serial: Date: Certificate: |  |  |  |  |  |  |  |
|  | Check mounting bolts to ensure tight. | - Check bolts are firm |  |  |  |  |  |  |  |  |
|  | Check Cable Entry and Gland | - Correct Certification - Hose Clamps Fitted and Secure |  |  |  |  |  |  |  |  |
|  | HAZARDAVERT FUNCTION TESTS |  |  |  |  |  |  |  |  |  |
|  | Ensure System is Powered ‘On’ | - PDS Bypass Switch Set to ‘On’ Mode. - Park Brake Engaged - POD + Proximity Modules Display as per right - Flasher/Siren shows steady white LED indication (indicates PDS in not bypassed) |  |  |  |  |  |  |  |  |
|  | Check releasing Park Brake activates fields | - Release Park Brake - POD + Proximity Modules Display as per right - PFER LED in Operator control station indicates fields enabled - Flasher/Siren shows no indication |  |  |  |  |  |  |  |  |
|  | Check “fields enabled relay” (PFER) prevents Shuttle car from moving if relay it does not operate. - Check by disconnecting wire 243 | - Remove wire 243 from terminals in PDS Main Controller box - Release Park Brake - POD + Proximity Modules Display as per right - PFER LED in Operator control station indicates fields disabled - Attempt to drive shuttle car. Shuttle car should not move - Replace wire 243 in terminals in PDS Main Controller box |  |  |  |  |  |  |  |  |
|  | Simulate park brake input failure. If one or both park brake inputs fail then: - PFER relay should prevent car from moving - PDS fields should turn to ‘ON’. i.e. PDS fields ‘ON’ is a failsafe condition. Note: the PDS requires a ‘low’ signal input condition from the park brake feedback to turn fields ‘ON’ | - Place a PDS cap lamp in the Shuttle car PDS Stop Zone - Unplug Park Status J80 phoenix plug from Strata main controller board - Release Park Brake - POD + Proximity Modules Display as per right - PFER LED in Operator control station indicates fields enabled - Attempt to drive shuttle car. Shuttle car should not move - Replace wire 24VI in terminals in PDS Main Controller box |  |  |  |  |  |  |  |  |  |
|  | Simulate mode sense valve failure If one or both park brake inputs fail then: - PFER relay should prevent car from moving - PDS fields should turn to ‘ON’. i.e. PDS fields ‘ON’ is a failsafe condition. Note: the PDS requires a ‘low’ signal input condition from the park brake feedback to turn fields ‘ON’ | - Place a PDS cap lamp in the Shuttle car PDS Stop Zone - Remove wire MSV2 from the mode sense valve - Release Park Brake - POD + Proximity Modules Display as per right - PFER LED in Operator control station indicates fields enabled - Attempt to drive shuttle car. Shuttle car should not move - Replace wire MSV2 on the mode sense valve switch. |  |  |  |  |  |  |  |  |
|  | Check Silent Zone is Active in Operators Cabin | - Place PAD inside Operators Cabin - Machine does not trip on Hazard Event |  |  |  |  |  |  |  |  |
|  | Check Warning Zone is Operational | - Place PAD on a Traffic Cone at 15m in front of shuttle car. - Release park brake & Drive slowly towards the PAD - POD + Proximity Modules Display as per right - Flasher/Siren shows Yellow Flashing Light (No Siren) - SC Slows to Warning Speed - Repeat Check at Rear of Shuttle Car |  |  |  |  |  |  |  |  |
|  | Check Hazard Zone is Operational | - Continue driving slowly towards PAD. - POD + Proximity Modules Display as per right - Flasher/Siren shows Red + Yellow Flashing Light - Flasher/Siren Sounds the Siren - SC Stops - Repeat Check at Rear of Shuttle Car |  |  |  |  |  |  |  |  |
|  | Check 2nd Stop Feature is Operational | - Remove K1 Relay in Main Enclosure. - Place PAD on a Traffic Cone at 15m in front of shuttle car. - Release park brake & Drive slowly towards the PAD - POD + Proximity Modules Display as per right - Flasher/Siren shows Red + Yellow Flashing Light - Flasher/Siren Sounds the Siren - SC trips the pilot |  |  |  |  |  |  |  |  |
|  | Check Hard Wired Bypass Switch is Operational | - Place PDS Bypass Switch into Bypass Mode - Place PAD on a Traffic Cone at 15m in front of shuttle car. - Release park brake and drive towards the PAD - POD + Proximity Modules Display as per right (System Off) - Flasher/Siren shows no indication - SC does not provide any intervention - Ensure shuttle car can move away from PAD. |  |  |  |  |  |  |  |  |
|  | Place PDS back into ‘On’ Condition and ensure PDS returns to Normal State | - PDS Bypass Switch Set to ‘On’ Mode. - POD + Proximity Modules Display as per right - Flasher/Siren shows no indication |  |  |  |  |  |  |  |  |
|  | Check Remote Stop Function is operational | - Press any 2 buttons on test PAD whilst standing outside of the hazard zone - Cap lamp should alarm in hazard - POD + Proximity Modules Display as per right - Flasher/Siren shows Red + Yellow Flashing Light - Flasher/Siren Sounds the Siren - UV to shuttle car trips - SC Stops | No lights on POD |  |  |  | No lights on Generators |  |  |  |
|  | Check Communication Check Operational | - Press S1 on Test PAD - POD + Proximity Modules Display as per right - Flasher/Siren shows no indication | FLASH TWICE |  |  |  | FLASH TWICE |  |  |  |
|  | Check Shrink Fields are Disabled | - With PAD in Warning Zone, Press S1 button - Zone Sizes do not change |  |  |  |  |  |  |  |  |
|  | Check System Bypass is operational | - With PAD in Hazard Condition, press buttons in sequence S1, S2, Menu - POD lights alternate in Ping/Pong Mode - PAD Continues to Alarm - SC Stops Alarming - SC can move | PING | PONG |  |  |  |  |  |  |
|  | Check POD Communication Error is operational | - Disconnect Comms cables from POD - POD Displays as per right |  |  |  |  |  |  |  |  |
|  | Check PM A1 Failure Detect is Operational | - Disconnect the Comms Line to PM A1 - POD + Proximity Modules Display as per right - Flasher/Siren shows Red + Yellow Flashing Light - Flasher/Siren Sounds the Siren - SC Cannot be moved |  |  |  |  | BLINKING Note: you made need to cycle power to get generator LEDs to faults |  |  |  |
|  | Check PM A2 Failure Detect is Operational | - Disconnect the Comms Line to PM A2 - POD + Proximity Modules Display as per right - Flasher/Siren shows Red + Yellow Flashing Light - Flasher/Siren Sounds the Siren - SC Cannot be moved |  |  |  |  | BLINKING Note: you made need to cycle power to get generator LEDs to faults |  |  |  |
|  | Check PM B1 Failure Detect is Operational | - Disconnect the Comms Line to PM B1 - POD + Proximity Modules Display as per right - Flasher/Siren shows Red + Yellow Flashing Light - Flasher/Siren Sounds the Siren - SC Cannot be moved |  |  |  |  | BLINKING Note: you made need to cycle power to get generator LEDs to faults |  |  |  |
|  | Check PM B2 Failure Detect is Operational | - Disconnect the Comms Line to PM B2 - POD + Proximity Modules Display as per right - Flasher/Siren shows Red + Yellow Flashing Light - Flasher/Siren Sounds the Siren - SC Cannot be moved |  |  |  |  | BLINKING Note: you made need to cycle power to get generator LEDs to faults |  |  |  |

| SHUTTLE CAR SAT FORM (Site Acceptance Test) – 3-Monthly & Commissioning |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SC MEASURED FIELDS - WARNING ZONE |  |  | SC MEASURED FIELDS - DANGER ZONE |  |  | EQUIPMENT VERSIONS |  |  |
| Position | Min / Max Field Size | Measured Field | Position | Min / Max Field Size | Measured Field | Equipment | Serial # | Firmware |
| AA | 6.0-9.0 |  | A | 3.0 – 6.0 |  | PM A1 |  |  |
| PP | 6.0-9.0 |  | P | 3.0 – 6.0 |  | PM A2 |  |  |
| OO | 6.0-9.0 |  | O | 3.0 – 6.0 |  | PM B1 |  |  |
| BB | <8 |  | B | 1.0 – 3.0 |  | PM B2 |  |  |
| CC | <8 |  | C | 0.5 – 3.0 |  | Display POD |  |  |
| DD | <8 |  | D | 0.5 – 3.0 |  | Flasher | N/A |  |
| EE | <8 |  | E | 0.5 – 3.0 |  | Bypass Switch | N/A |  |
| FF | <8 |  | F | 0.5 – 3.0 |  | Controller Box (IM) |  |  |
| GG | 6.0-9.0 |  | G | 3.0 – 6.0 |  | Controller WiFi |  |  |
| HH | 6.0-9.0 |  | H | 3.0 – 6.0 |  | IP Address |  |  |
| II | 6.0-9.0 |  | I | 3.0 – 6.0 |  | Visible on Server? |  |  |
| JJ | <8 |  | J | 0.5 – 3.0 |  |  |  |  |
| KK | <8 |  | K | 0.5 – 3.0 |  |  |  |  |
| LL | <8 |  | L | 0.5 – 3.0 |  |  |  |  |
| MM | <8 |  | M | 0.5 – 3.0 |  | Test PAD ID |  |  |
| NN | <8 |  | N | 1.0 – 2.5 |  | Test PAD SW |  |  |

FIELD MEASURE POSITIONS

Controller Box

Generator

K-Box

Display POD

Flasher

Warning Zone

Danger Zone

Legend

Note:

- The MAXIMUM field can be exceeded in order to satisfy the minimum of all required fields.

- The danger zone takes PRIORITY in terms of field setup; the warning zone will be determined by the danger zone.

| Acceptance by Mine | Name |  |  | Position |  | Signed |  | Date |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Acceptance by Mine | Name |  |  | Position |  | Signed |  | Date |  |
| Acceptance by Strata | Name |  |  | Position |  | Signed |  | Date |  |
| Are Field Sizes Correct? | YES | NO |  |  |  |  |  |  |  |